# Calculation of palm tree décor for Polygonal 30 m

# Loads

# a- Dead Loads .

# b- Life loads.

# c- wind vibration effect.

# a-Dead loads.

## Design factors

# - Palm branch length average  = 4,2 m  , Width = 85,0 cm

# Total weight of branch = 60,0 kg.

# composed from 3 sectors as design, A,B,C.

# - Specific Weight of  branch

# -Sector 1 ( 0.9 m )

# 1-Galvanized steel tube ( a) D=50mm ,thickness 5mm, length = 50 cm welded to Galv, steel  angle 60x60x6mm which is fixed to pole surface by welding  with Galvanized steel angle 70x70x7mm H=150mm .( see detail )

2- The steel tube (a)is fixed with branch inside  Galvanized steel tube (b)  D= 60 mm , thickness 5mm ,length = 80,0 cm imbedded in the branch GRP base

3- High pressure  Propylene tube (c) D= 50mm length 1,5 m imbedded 28 cm inside branch steel tube (b).

4-  Bamboo wood rods (d) one  D = 40mm length 4,0m and 2D= 20mm length 4,0 m  imbedded 28,0 cm in Propylene tube (d) .

5- Polyester GRP (e) reinforced with fiberglass mesh 10 layers 8mm thick.   (  length = 0,9 m )

Measured weight of sector 1= 15 ,00 kg

-Sector 2 ( 1.3 m)

1-  Propylene tube ( c) length 132 cm .

2- Bamboo wood rods  (d)  132,0 cm .

3- Polyster GRP reinforced by 2 layers of Fiberglass fabric textile  yarn 400 kg/m3  and   10  layers of fiber mesh  300 kg/m3 round branch 10cm x 7 cm wide  (length  1,3 m)(f).

4- GRP branch leaves (g) V shape  40x40mm length 40 cm on both sides sewed with the main branch ,   thickness 3mm reinforced with 2 layers of fiber glass fabric yarn 400 kg/m3and 2 layers of fiberglass mesh 300 kg/m3. all fabricated round the bamboo rods.

,  Measured weight 11,0 kg

-Sector 3 ( 2,3 m)

1- Bamboo wood rods  (d)  2,3m.

2- Polyster GRP reinforced by 2 layers of Fiberglass fabric textile yarn 400 kg/m3  and   10  layers of fiber mesh  300 kg/m3 round branch 10cm x 7 cm wide  (length  1,0 m)(f).

3- GRP branch leaves (g) V shape  40x40mm length 40 cm on both sides  thickness 3mm reinforced with 2 layers of fiber glass fabric yarn 400 kg/m3and s lyers of fiberglass mesh 300 kg/m3. all fabricated round the bamboo rods.

Mesured weight =25,0 k                                                                        WEIGTT OF BRANCHES ( AVAREGE)

= 15,0 + 11,0+25,0= 51 kg

# No of branches = 36

Total vertical load of branches = 36 x 51,0 = 1836,0 kg

-Weight of steel pole and equipment = 4500 kg.   ( Given by manufacturer)

Pressure distributed on pole

= Pole weight + Branches weight  / pole  base cross section area

= (1836,0 + 4500,0   / 2,2 m2   = 2880,0  kg / m2

= 0,2880 kg / cm2    ( safe for the concrete base )

# b- Life Loads ( Wind Pressure )

DESIGN FACTORS

# - Overhaul thickness of branch all over the length = 0,5 cm  including polyester and fiber glass textile  400kg/m3 yarn skin and sub branches  30-40 cm .

-Monopole high = 30,0 m

-Palm branches  High = 27- 33 m ,  average High  H  = 6,0 m

- Branches are distributed on the pole circumference 6  branches in row. And average angle from horizontal vary from 30°-75°

- - Number of rows 6 ,  50 cm between.

- - total no of branches 36

- -   The  wind average  speed in Egypt vary from 3 m/ sec to 20 m/sec  ( 11-72 km/hr/) By weather department.

- For design : wind speed calculated 40 m/sec. ( 150 km/hr)

- Storm wind ( not registered in Egypt before)

- Air speed is assumed = 40 m/sec.

- - Design of wind speed = Vs = V  .S1 . S2. S3.

- S1 = 1  ( Topography factor )

- S2 = 0,9 ( Ground roughness )

- S3 = 1 ( Statical factor ).

- Vs =  36 m/sec.

Dynamic Wind pressure  Q= K Vs2

K = Constant = 0,613 .

Q= 0,613 x 36 2 = 794,4  N/m2

The wind force ( F ) is given by ,   F = Cpx  x Q  Where Cpx External pressure coefficient.  = 1,2 .

Therefore Wind Force ( F ) =- 1,2 x 794,4 = - 953 N/m2  .

Wind Force WI ten   in direction perpendicular to Palm surface

Wind loads :

wind pressure =- 953 N/m2

Area of Palm subjected = Palm Branches area  D x branch length  x percentage of branches area subjected to wind for distributed 6 branches in 6rows 50 cm apart. over the pole circumference , with the condition that 50 % of branches subjected  to head wind and 50% to tail wind:

Area of branches subjected to wind pressure;

Considering the following:

1- due to voids between leaves the effective area =65% of the blank areas.( Leaves void factor  Lv.= 0,65 )

2- In each row of branches the distribution of 6 branches over 360° on equal angles = 60°  between branches . so the effective area due to this = 70% .( Distribution factor Ds= 0,7)

3- the  subjected branches to wind blow is 3 branches because the  other 3 are shaded by the subjected branches and pole body .

( Shading factor Sh = 0.5).

4- Area reduction due to angle of inclination for rows vary from 30° to horizontal two rows (= 0,33 )  , 55° for 2 rows ( = 0, 71) and 75° for 2 rows( = 0,83).

The actual area subjected to wind blow :

a-12 Br, x 3,5 m(length) x 0,1 m (width) x 0,83 ( area of branch subjected to 75 °)x 0,5( Sh) x 0,7 (Ds) x 0,65 (Lv)

=7,9 m2.

b-12Br, x 4,0m x 0,1m 0,71(area of branches subjected to wind

55 °) x 0,5( Sh) x 0,7 (Ds) x 0,65 (Lv)

=7,75 m2.

c-12Br, x 4,5m x 0,1m 0,33( area of branches subjected to wind

30 °) x 0,5( Sh) x 0,7 (Ds) x 0,65 (Lv)

=4,05m2

Total area subjected to wind = 7,9 + 7,75 + 4,05 = 19.7 m2.

Forces on pole and moments on footing:

Force on pole top 5  m    = WI M1 = 953 N/m2 x 19,7m2 ( area subjected) x loss Factor due to air vortices of branch vibration under wind  Φ

#### Φ= 0,3=

#### WI M1 = 953 N/m2 x 19,7m2 x 0,3

= 5632,0  N

= 563 ,2   kg  (aprox)

Shear forces on pole base:

- Shear forces = 536,2 kg.

- No of bolts M20( D=20mm ) = 26

- Shear stress for bolt = 20,6 kg.         ( SAFE)

Tension forces by moment  force of wind pressure on pole base:

Subjected on half of the base connecting steel ring fixed with 26 bolts M20 mm .

Tension due to momentum force on pole base WI M

WI M1  = ( wind pressure on branches ) 563,2 kg  x (Pole high)  25 m =  14080,0  kg.m .

Tension load on fixation bolts for 13 bolt subjected  = 14080,0 /    kg.

=for bolts M20 mm carrying tension  1083,0  kg each  ,

Cross section of bolt area = 314,0 mm2

Tension force = 1083,0 / 314,0 = 3,5 kg/mm2  ( SAFE.)

The décor of pole is safe for dead and life loads

# ALCULATION FOR PALM FIXATION

# WITH POLYGRAM BODY

# 1- Without wire for the most effected branches of the first raw:

# - Weight of branch = 60 kg.

- Distance from CG of the branch to connection for

4,5m branch= 1,8 m

# - Maximum wind pressure = 96,0 Kg /m2

# - Angle of inclination = 30°

# - Subjected area to wind pressure = 4,5m x 1,0 m x 0,33(area subjected by angle 30°) x  0,5(Sh) x 0,7 ( Ds) x 0,65 ( Lv) = 0,34 m2

Up lift by wind pressure :

- Area subjected x wind force

-= 0,34 x  96,0  = 32,6 kg.

Safety factor for wind vibration = 1,5

- Maximum force = 32,6 x 1,5 = 48,9 kg.

Shear force on angle of the fixation column:

-= Weight of branch = 60,0 Kg.

- Area of weld  =  0,12 m x 0,08 ( weld width) = 0,0096m2

= 96,0 kg/cm2 = 0,96 kg/mm2. (  Safe) .

Moment force on column welding = weight of branch x L ( distance from column base to branch c.g .

= 60,0 kg x 1,8 m= 108,0 kg.m.( wind pressure if exist is negative.

Tensile force subjected on column base= 108,0 kg / weld area = 108,0 / 3,14 x 5,0 x 0,6 =

108,0 / 9,42 cm2 = 11,4 kg/cm2  .

Safty factor  for vibration =1,5

Tensile force on column base = 1,5 X 11,4 = 17,1kg/cm2

( safe as steel column and welding )

Effect of aging due to wind and weather

The most effective factor on branches fixatin is the  vibration effect due to aging which make internal cracks , and by time the cracks incrase and weaken the structure and my lead to failure ;

For this reason we add Galvanized steel robe diameter 5 mm and tension capability = 500 kg,to be attached 1,5 m from fixation and by angle 30° with the branch. Which carry 30% of the forces and decrease the angle of fluctuations due to wind .

Also the fixation must be revised due to the previous effects  at least every 6 months  for welding and fixation angles. And change wire if happen any corrosion or damage.

Eng, Yehia Younes Sherif

Master for Mechanical and avionics engineering.

# Signature